## Reports

## Additional Information

April 1994

Versions: 	InRail 03.00.00.00

InRoads 05.00.01.00

InXpress 02.00.01.00

SiteWorks 02.00.01.00

CogoWorks 02.00.01.00

Warranties and Liabilities

All warranties given by Intergraph Corporation about equipment or software are set forth in you purchase contract, and nothing stated in, or implied by, this document or its contents shall be considered or deemed a modification or amendment of such warranties.

The information and the software discussed in this document are subject to change without notice and should not be considered commitments by Intergraph Corporation.  Intergraph Corporation assumes no responsibility for any error that may appear in this document.

The software discussed in this document is furnished under a license and may be used or copied only in accordance with the terms of this license.

No responsibility is assumed by Intergraph for the use or reliability of software on equipment that is not supplied by Intergraph or its affiliated companies.

Trademarks

CLIPPER, CLIX, InterAct, and Intergraph are registered trademarks of Intergraph Corporation.  InRoads, InRail, InXpress, SiteWorks, CogoWorks, DraftWorks, MasterWorks, AEC Group, and the AEC Group logo are trademarks of Intergraph Corporation.  MicroStation is a trademark of Bentley Systems Inc., an Intergraph affiliate.  UNIX is a trademark of AT&T Bell Laboratories.  IBM is a registered trademark of International Business Machines Corporation.  All other brands and product names are trademarks of their respective owners.

Copyright

Ó 1994 Intergraph Corporation

All Rights Reserved

Including software, file formats, and audiovisual displays; may be used pursuant to applicable software license agreement; contains confidential and proprietary information of Intergraph and/or third parties which is protected by copyright and trade secret law and may not be provided or otherwise made available without proper authorization.

RESTRICTED RIGHTS LEGEND

Use, duplication, or disclosure by the government is subject as set forth in subparagraph (c) (1) (ii) of The Rights in Technical Data and Computer Software clause at DFARS 252.227-7013 or subparagraphs (c) (1) and (2) of Commercial Computer Software - Restricted Rights at 48 CFR 52.227-19, as applicable.

Unpublished - rights reserved under the copyright laws of the United States.

Intergraph Corporation

Huntsville, Alabama 35894-0001

Table of Contents

General Notes 	  1

Table 1 - Report Associations for geo.dba 	  2

Report Definitions and Examples for geo.dba 	  4

Area 	  4

List 	  4

Traverse 	  5

Clearance 	  6

Center Stakeout 	  7

Radial Stakeout 	  8

Station Single 	  9

Station Base 	  10

Station Offset 	  12

Horizontal Alignment 	  14

Vertical Alignment 	  16

Track Elements 	  17

Track Turnouts 	  17

Table 2 - Report Associations for rwy.dba, vol.dba and xsec.dba 	  19

Report Definitions and Examples for rwy.dba   	  20

Profile Plot 	  20

Roadway Modeler 	  21

Slope Staking Report 	  22

Template Reports 	  23

Report Definitions and Examples for vol.dba   	  26

Multiple Material Report  	  26

Basic End-Area Report 	  27

Topsoil Report 	  28

Forced Balance 	  29

Report Definitions and Examples for xsec.dba  	  30

Cross-Section Reports 	  30

Cross-Section Export Report 	  31

Binary File Structure - geo 	  32

Binary File Structure - rwy 	  42

Binary File Structure - vol 	  43

Binary File Structure - xsec 	  44

Reports

General Notes

All of the reports created within this command make use of the DB Access library files and the templates within those libraries.  There are four DB Access library files delivered with the product.  These files are:  geo.dba, rwy.dba, vol.dba and xsec.dba.

All the reports listed under Type except for Other are based on the active geometry project file and use the geo.dba library file.  When these reports are executed, a binary file is created internally and then the ASCII format is generated.  The binary file is discarded unless it has been requested to be written to disk.

Use of the Type of  Other provides access to formatting the binary files created when the Roadway Modeler, End-Area Volume and Cross-Section commands are processed.  When these three commands are processed, there is a toggle to have a binary DB Access report file created.  This binary file has to be created when the command is processed.  Once the binary file is created, it can then be taken into Utilities > Reports and using a Type of Other, it can be formatted into an ASCII report file.  The DB Access library files and the commands match up as follows:

rwy.dba - Roadway Modeler	vol.dba - End-Area Volume	xsec.dba - Cross-Sections

The "piped" and "comma" template formats will create an ASCII formatted report of all the data contained in the binary file for that report type with the pipe ( | ) or comma ( , ) delimeter between the fields.  This allows the ASCII report to be imported into a spreadsheet like Excel and to use the delimeter for creating columns in the spreadsheet.  The spreadsheet program can then be used to customize reports.

The stations that are reported on will vary based on the Station Lock setting.

Auto Plot lock - if on, graphics are displayed to the design file showing the reported results.  If Write lock is on, the graphics are written to the design file.  If off, graphics are temporary display only.

Multiple alignments can be specified with in the Include Alg field with wildcards or in a list separated by commas, (ie  lot*   elm,willow).  Note: There are no spaces in a list of alignment names.

If there is a mismatch in the report type and the template selected, you may get garbage in the report or zeros.  Check the Type and Template selected.

If the report parameters on the dialog box are not set correctly for the specified alignment, may get the message, Nothing to Report On.  Check and correct settings.

ASCII files created with the File > Save As command on the Reports dialog box will have a carriage return at the point where the text wraps in the display window.  ASCII files created by toggling on the ASCIII option under Output will give the full length of the line and then a carriage return.

Table 1 - Report Associations for geo.dba

| Type | .dba library | .dba template | description |
| --- | --- | --- | --- |
| Area | geo.dba | AREA_REPORT | closed area report |
|  |  | COMMA_AREA | default Area comma |
|  |  | PIPED_AREA | default Area piped |
| List | geo.dba | LIST_COORDINATES | list coordinates |
|  |  | COMMA_LIST | default List comma |
|  |  | PIPED_LIST | default List piped |
| Traverse | geo.dba | TRAVERSE_ANG_RPT | traverse angle report |
|  |  | TRAVERSE_BRG_RPT | traverse bearing report |
|  |  | COMMA_TRAVERSE | default Traverse comma |
|  |  | PIPED_TRAVERSE | default Traverse piped |
| Clearance | geo.dba | CLEARANCE_REPORT | clearance report |
|  |  | COMMA_CLEAR | default Clearance comma |
|  |  | PIPED_CLEAR | default Clearance piped |
| Center Stakeout | geo.dba | CENTERLINE_STAKEOUT | centerline stakeout report |
|  |  | COMMA_CL_STAKE | default CL Stake comma |
|  |  | PIPED_CL_STAKE | default CL Stake piped |
| Radial Stakeout | geo.dba | RADIAL_STAKEOUT | radial stakeout report |
|  |  | COMMA_RADIAL_STAKE | default Radial Stake comma |
|  |  | PIPED_RADIAL_STAKE | default Radial Stake piped |
| Station Single | geo.dba | SE_SINGLE_80 | station elev single |
|  |  | SE_SINGLE_GRD_80 | station elev grade single |
| Station Base | geo.dba | SOE_BASE_80 | base stn & offsets |
|  |  | SOE_BASE_80_XYZ | base stn & offsets w/ xyz |
| Station Offset | geo.dba | SOE_DEFAULT * | default SOE report |
|  |  | SOE_OFFSET_80 | stn & offsets |
|  |  | SOE_OFFSET_80_XYZ | stn & ofsets w/ xyz |
|  |  | SOE_OFFSET_GRD_80 | stn & offsets w/ grade |
| Station Offset | geo.dba | SO_OFFSET_132 | stn & offsets 132 columns |
|  |  | COMMA_SOE * | default SOE comma |
|  |  | PIPED_SOE * | default SOE piped |
| Horz Alignment | geo.dba | HORIZ_ALIGN_132 | horizontal align 132 columns |
|  |  | HORIZ_ALIGN_A_80 | horiz align opt a |
|  |  | HORIZ_ALIGN_B_80 | horiz align opt b |
|  |  | COMMA_HORIZ | default horiz align comma |
|  |  | PIPED_HORIZ | default horiz align piped |
|  |  | TRACK_ET | track et |
| Vert Alignment | geo.dba | VERT_ALIGNMENT | vertical alignment report |
|  |  | COMMA_VERT | default Vert Align comma |
|  |  | PIPED_VERT | default Vert Align piped |
| Track Elements | geo.dba | none at this time |  |
| Track Turnouts | geo.dba | none at this time |  |

Notes:

The three reports marked with an asterick (*), SOE_DEFAULT, COMMA_SOE and PIPED_SOE can be used with any of the Station type reports.  Also, most of the templates listed for the Station type reports can be used interchangeably within the Station type reports.  When crossing the templates with the other Station type reports, some of the data fields may not be reported or may be reported as zero.

The following templates appearing in the geo.dba library are used for formatting tables created with the View > Annotation > Horizontal command and wysiwyg settings.

TABLES_ALL	PIPED_TABLE_CURVE	COMMA_TABLE_CURVE

TABLES_CURVES	PIPED_TABLE_LINE	COMMA_TABLE_LINE

TABLES_LINES	PIPED_TABLE_PNT	COMMA_TABLE_PNT

TABLES_SPIRALS	PIPED_TABLE_SPIRAL	COMMA_TABLE_SPIRAL

Report Definitions and Examples for geo.dba

Area 	-  Reports on the calculated areas of closed figures.  Areas are calculated in square feet (meters) and acres (hectares).  The figure(s) reported on are set in the Include Alg field and additional restraints can be defined with the selection criteria.

Type:	Area	Library:	geo.dba	Template:	AREA_REPORT

AREA Report

PROJECT: sub                       Page   1 of   1

18-APR-1994

09:45 AM

LOT              AREA(sq.ft.)  AREA(acres)  msg

----             ------------ ------------  ---

lot1               14869.4161       0.3414

lot2               15066.2289       0.3459

lot3               17332.2247       0.3979

lot4               16536.7495       0.3796

lot5               19012.9306       0.4365

lot6               17799.5397       0.4086

lot7               34643.0666       0.7953

List	-  Reports on point coordinate data for all or a portion of the active geometry project.  Point coordinate data is reported on for the alignment(s) identified in the Include Alg field.  The selection criteria can be used to identify a subset of the alignment data to be reported on.

Type:	List	Library:	geo.dba	Template:	LIST_COORDINATES

LIST COORDINATES Report

April 18, 1994                                     Page    1 of    1

Project: sub

Alignment: boundary

NAME         NORTHING      EASTING     ELEVATION   PREFERENCE   des

-------     ----------    ----------   ----------  ----------  ------

1000        11021.7048    10644.3473     0.0000     default

1001        11160.6153    10711.9205     0.0000     default

1002        11497.6657    11267.1298     0.0000     default

1003        11506.6825    11358.4919     0.0000     default

1004        11405.5284    11487.9779     0.0000     default

1005        10672.0881    11855.1640     0.0000     default

1006        10515.4410    11850.2172     0.0000     default

1007        10297.3705    11433.4609     0.0000     default

1008        10010.1124    11279.1023     0.0000     default

1009         9953.9917    11165.1288     0.0000     default

1010        10055.2937    11102.8743     0.0000     default

1011        10443.7646    10993.0215     0.0000     default

1012        11021.7049    10644.3474     0.0000     default

1000        11021.7048    10644.3473     0.0000     default

Traverse	-  Reports on the distances, directions, and angles between consecutive points of any figure.  Within this command, the consecutive points are the points of curvature (PCs, PTs, TSs, SCs, CSs, and STs) and angle points (PIs without an associated arc).  The figure reported on is the one defined in the Include Alg field on the Reports dialog box.  You can modify the report data by setting the selection criteria.

Type:	Traverse	Library:	geo.dba	Template:	TRAVERSE_BRG_RPT

TRAVERSE Report

18-APR-1994                                             Page   1 of   1

09:45 AM

Project: sub

Alignment: elm

BS      OCCUPIED      FS          BEARING          DISTANCE

---------  ---------  ---------      ----------       ----------

300        301      S 61^20'45.50" E       51.0365

300        301        500      N 28^39'14.50" E      400.0000

301        500        302      S 29^48'02.01" E      400.0000

500        302        303      N 60^11'33.44" E      293.6278

TRAVERSE Report

Page   1 of   1

Project: sub

Alignment: lot4

BS      OCCUPIED      FS          BEARING          DISTANCE

---------  ---------  ---------      ----------       ----------

7          8       N 58^44'21.70" E       85.0000

7          8          12      S 31^15'38.30" E      195.6282

8          12         11      S 60^11'33.44" W       85.0273

12         11         7       N 31^15'38.30" W      193.4718

11         7          8       N 58^44'21.70" E       85.0000

Clearance	-  Reports on the station and offset relationship between the active horizontal alignment and the alignment(s) specified in the Include Alg field on the Reports dialog box.  You can modify the report data by setting the selection criteria.

Type:	Clearance	Library:	geo.dba	Template:	CLEARANCE_REPORT

CLEARANCE REPORT

PROJECT: sub                                      Page   1 of   1

TRAVERSE ALIGN: boundary

April 18, 1994       09:45 AM

OFFSET  OFFSET     OFFSET      OFFSET    BASELINE             RADIAL

ALIGN  PT NAME    NORTHING    EASTING     STATION  OFFSET    DIRECTION

-------  ------   ---------   ---------  ---------- ------  ------------

willow   600     11030.663   10875.651    2+27.00  196.05 S 31^15'38.30" E

willow   601     10916.312   10898.284   39+36.71  162.99 N 58^53'50.00" E

willow   602     11008.455   11348.961   13+52.93  302.07 S 63^24'21.00" W

willow   501     10696.326   11011.062   36+90.09  145.91 N 58^53'50.00" E

willow   502     10612.408   11088.580   35+78.20  168.94 N 58^53'50.00" E

willow   603     10307.065   10758.027   34+15.56 -263.32 S 74^12'36.10" W

willow   604     10368.656   11203.792   33+53.53  182.38 N 74^12'36.10" E

willow   605     10244.079   11221.005   26+54.92  161.92 N 61^44'54.80" W

Center Stakeout	-  Reports the centerline staking data from the active horizontal alignment to stations along the alignment(s) specified in the Include Alg field on the Reports dialog box.  In the Reports dialog box the Initial Stn, Final Stn, Increment and Offset fields are defined for the specified alignment.  If an offset is defined, the offset if from the specified (centerline) alignment.  The staking data is reported from the active horizontal alignment to the offset distance from the centerline of the specified alignment.  You can modify the report data by setting the selection criteria.

Type:	Center Stakeout	Library:	geo.dba	Template:	CENTERLINE_STAKEOUT

CENTER STAKEOUT Report

PROJECT: sub                                       Page   1 of   1

TRAVERSE ALIGN: boundary

April 18, 1994       09:45 AM

OFFSET FROM

CENTERLINE CENTERLINE   BS       OC       FS STN    ANGLE RIGHT  DISTANCE

----------  -------- -------- -------- ----------  ------------  --------

elm         0.000   1001     1000     7+35.28   282^37'16.07"   116.96

elm         0.000   1001     1000     7+86.32   290^07'22.67"    67.26

elm         0.000   1001     1000     8+00.00   294^40'05.18"    54.45

elm         0.000   1001     1000     9+00.00   55^41'58.32"     60.08

elm         0.000   1011     1012    10+00.00   301^13'05.36"   158.77

elm         0.000   1000     1001    11+00.00   277^06'45.30"   226.30

elm         0.000   1000     1001    11+94.41   261^27'07.45"   289.60

elm         0.000   1000     1001    12+00.00   260^39'04.73"   293.42

elm         0.000   1000     1001    13+00.00   249^21'02.40"   369.55

elm         0.000   1001     1002    14+00.00   318^37'02.96"   336.76

elm         0.000   1001     1002    14+88.04   306^12'54.77"   278.70

Radial Stakeout	-  Reports the radial stake-out data from the active horizontal alignment to the alignment(s) specified in the Include Alg field on the Reports dialog box.  You can modify the report data by setting the selection criteria

Type:	Radial Stakeout	Library:	geo.dba	Template:	RADIAL_STAKEOUT

RADIAL STAKEOUT Report

April 18, 1994                                         Page   1 of   1

PROJECT: sub

TRAVERSE ALIGN: boundary

ALIGN         BS         OC         FS        ANGLE RIGHT  DISTANCE

---------   ---------   ---------  ---------   ------------  ---------

elm         1001       1000        300     282^37'16.07"   116.9608

elm         1001       1000        301     290^07'22.67"    67.2612

elm         1000       1001        500     170^38'17.42"   271.8383

elm         1000       1001        302     261^27'07.45"   289.5988

elm         1001       1002        303     306^12'54.77"   278.7003

willow       1000       1001        600     282^29'53.13"   209.0342

willow       1011       1012        601     323^38'34.74"   274.9391

willow       1003       1004        602     251^17'54.53"   420.7051

willow       1010       1011        501     199^52'32.52"   253.2053

willow       1010       1011        502     225^19'38.07"   193.8353

willow       1010       1011        603      75^36'09.97"   271.8622

willow       1010       1011        604     305^24'13.24"   223.7535

willow       1006       1007        605     193^32'23.70"   219.0380

Station Single	-  Reports on the stations and elevations or the stations, elevations and grades along the specified alignment in the Include Alg field.  The elevations and grades are based on the active vertical alignment for the specified horizontal alignment.  If there is no vertical alignment, the elevations and grades are computed from the elevations of the horizontal alignment PI points.  In the Reports dialog box the Initial Stn, Final Stn, Increment and Offset fields are defined for the specified alignment.

Type:	Station Single	Library:	geo.dba	Template:	SE_SINGLE_GRD_80

April 18, 1994                         PAGE   1 OF   1

STATION ELEVATION GRADE (SINGLE) REPORT

BASELINE            BASELINE        BASELINE

STATION               ELEV           GRADE

(   new    )        (   vert   )        (%)

229+12.92             961.00           2.74

229+53.39             962.11           2.74

230+00.00             963.39           2.74

231+03.39             966.22           2.74

232+14.00             969.25           2.74

235+00.00             972.41          -0.53

238+14.00             965.10          -4.12

240+00.00             957.44          -4.12

241+04.00             953.15          -4.12

241+73.20             950.48          -3.60

243+23.20             945.94          -2.46

244+04.00             944.21          -1.84

245+00.00             942.44          -1.84

250+00.00             933.23          -1.84

250+53.46             932.25          -1.84

253+58.10             926.64          -1.84

255+00.00             925.05          -0.40

257+16.68             926.58           1.81

258+69.90             930.55           3.37

260+00.00             934.93           3.37

260+79.03             937.59           3.37

264+02.00             948.46           3.37

265+00.00             951.62           3.07

270+00.00             963.20           1.56

272+02.00             965.74           0.95

275+00.00             968.57           0.95

280+00.00             973.34           0.95

281+12.66             974.41           0.95

285+00.00             978.10           0.95

287+00.00             980.00           0.95

288+10.41               0.00           0.00

Station Base	-  Reports on the stations, offsets and elevations from the active alignment to the alignment specified in the Include Alg field.  The stationing is incremented along the baseline (active) alignment.  In the Reports dialog box the Initial Stn, Final Stn and Increment are defined for the active alignment.  The specified alignment elevation is from the active vertical alignment of the specified horizontal alignment.  The distances to the specified alignment are all shown as positive distances.

The following example is a Station Base report with the offset from the active alignment at zero.   There is no offset from the active alignment so the offset distance is directly determined radially to the specified alignment.  Stationing is incremented along the active (baseline) alignment.

Type:	Station Base	Library:	geo.dba	Template:	SOE_BASE_80

April 18, 1994                                    Page   1 of   1

STATION BASE REPORT

ACTIVE ALIGN      OFFSET FROM        SPECIFIED     DISTANCE TO

STATION        ACTIVE ALIGN       ALIGN ELEV   SPECIFIED ALIGN

( traverse )     ( traverse )      (cntr_vert )    ( cntrline )

2+00.00             0.00            926.35         103.27

3+00.00             0.00            930.45         185.33

3+50.82             0.00            932.37         217.90

4+00.00             0.00            927.44         169.33

5+00.00             0.00            930.86         129.02

6+00.00             0.00            934.88          53.34

7+00.00             0.00            951.18         148.14

8+00.00             0.00            968.80         292.19

8+01.12             0.00            968.92         292.88

9+00.00             0.00            950.69         146.47

10+00.00            0.00            960.60         138.56

11+00.00            0.00            971.57          81.71

12+00.00            0.00            977.53           1.40

13+00.00            0.00            978.10          84.50

13+09.47            0.00            978.14          92.37

Station Base	-  continued

The following example is a Station Base report with the offset at fifty feet to the right from the active alignment.   The offset from the active alignment is set and then the distance is determined radially to the specified alignment.  Stationing is incremented along the active (baseline) alignment.

Type:	Station Base	Library:	geo.dba	Template:	SOE_BASE_80

April 18, 1994                                    Page   1 of   1

STATION BASE REPORT

ACTIVE ALIGN      OFFSET FROM        SPECIFIED     DISTANCE TO

STATION        ACTIVE ALIGN       ALIGN ELEV   SPECIFIED ALIGN

( traverse )     ( traverse )      (cntr_vert )    ( cntrline )

2+00.00            50.00            926.35         153.27

3+00.00            50.00            930.45         235.33

3+50.82            50.00            932.37         267.90

4+00.00            50.00            927.44         219.33

5+00.00            50.00            930.86         179.02

6+00.00            50.00            934.88         103.34

7+00.00            50.00            951.18          98.14

8+00.00            50.00            968.80         242.19

8+01.12            50.00            968.92         242.88

9+00.00            50.00            950.69          96.47

10+00.00           50.00            960.60          88.56

11+00.00           50.00            971.57          31.71

12+00.00           50.00            977.53          51.40

13+00.00           50.00            978.10         134.50

13+09.47           50.00            978.14         142.37

NOTE:	A Trouble Report (TL#19276) has been logged against this report to change the offset to be from the specified alignment, not the active alignment.

Station Offset	-  Reports on the stations, offsets and elevations from the specified (offset) alignment as defined in the Include Alg field to the active horizontal alignment.  The stationing is incremented along the specified (offset) alignment.   In the Reports dialog box the Initial Stn, Final Stn and Increment are defined for the specified alignment.  The specified alignment elevation is from the active vertical alignment of the specified horizontal alignment.

The following example is a Station Offset report with the offset from the specified alignment at zero.  There is no offset from the specified alignment so the offset distance is directly determined radially to the active alignment.  Stationing is incremented along the specified (offset) alignment.

Type:	Station Offset	Library:	geo.dba	Template:	SOE_OFFSET_80

April 18, 1994                                       Page   1 of   1

STATION OFFSET REPORT

SPECIFIED ALIGN   DISTANCE FROM      SPECIFIED      OFFSET FROM

STATION         ACTIVE ALIGN       ALIGN ELEV   SPECIFIED ALIGN

( cntrline )      ( traverse )      (cntr_vert )    ( cntrline )

0+00.00           -76.23            925.00           0.00

1+00.00          -139.69            928.17           0.00

1+66.96          -136.02            930.30           0.00

2+00.00          -122.15            931.34           0.00

3+00.00           -61.49            934.52           0.00

3+00.30           -61.28            934.52           0.00

4+00.00            21.64            939.35           0.00

4+06.67            27.78            939.79           0.00

5+00.00           113.98            947.53           0.00

5+00.30           114.25            947.56           0.00

5+92.25           144.16            956.62           0.00

6+00.00           143.71            957.39           0.00

7+00.00           111.89            967.25           0.00

7+03.16           110.15            967.56           0.00

7+23.23            98.12            969.44           0.00

8+00.00            49.05            974.87           0.00

9+00.00           -14.87            977.74           0.00

9+03.16           -16.88            977.75           0.00

10+00.00          -78.78            978.07           0.00

Station Offset	- continued

The following example is a Station Offset report with the offset at fifty.  The fifty foot offset from the specified alignment is determined and then the offset distance is determined radially to the active alignment.  Stationing is incremented along the specified (offset) alignment.

Type:	Station Offset	Library:	geo.dba	Template:	SOE_OFFSET_80

April 18, 1994                                     Page   1 of   1

STATION OFFSET REPORT

SPECIFIED ALIGN   DISTANCE FROM      SPECIFIED      OFFSET FROM

STATION         ACTIVE ALIGN       ALIGN ELEV   SPECIFIED ALIGN

( cntrline )      ( traverse )      (cntr_vert )    ( cntrline )

0+00.00           -37.58            925.00          50.00

1+00.00          -101.05            928.17          50.00

1+66.96           -89.55            930.30          50.00

2+00.00           -77.99            931.34          50.00

3+00.00           -27.45            934.52          50.00

3+00.30           -27.27            934.52          50.00

4+00.00            41.83            939.35          50.00

4+06.67            46.94            939.79          50.00

5+00.00           133.14            947.53          50.00

5+00.30           133.41            947.56          50.00

5+92.25           194.12            956.62          50.00

6+00.00           193.56            957.39          50.00

7+00.00           153.79            967.25          50.00

7+03.16           151.61            967.56          50.00

7+23.23           136.57            969.44          50.00

8+00.00            87.50            974.87          50.00

9+00.00            23.59            977.74          50.00

9+03.16            21.57            977.75          50.00

Horz Alignment	-  Reports on the horizontal alignment specified in the Include Alg field.  The horizontal data reported on is derived from the settings in selection criteria which include name, preference, description and fence.

Type:	Horz Alignment	Library:	geo.dba	Template:	HORIZ_ALIGN_A_80

April 18, 1994                                                PAGE   1 OF   1

Project: lab

Alignment: new

Description: New alignment US_old

STATION     PT                       NORTH(Y)        EAST(X)

229+12.92  POB                      10983.8800      10397.6000

TL=     40.4745       BEARING= N 24^06'37.11" E

229+53.39  TS                       11020.8235      10414.1336

229+53.39  TS                       11020.8235      10414.1336

LS=    150.0000            SD=  4^17'49.86"

U=    100.0295             V=     50.0268

X=    149.9156             Y=      3.7485

231+03.39  SC                       11156.1294      10478.7948

231+03.39  SC                       11156.1294      10478.7948

LC=   1069.8054            CD= 61^17'43.20"

RC=   1000.0000            DC=  5^43'46.48"

241+73.20  CS                       11680.3767      11353.1945

241+73.20  CS                       11680.3767      11353.1945

LS=    150.0000            SD=  4^17'49.86"

U=    100.0295             V=     50.0268

X=    149.9156             Y=      3.7485

243+23.20  ST                       11673.6584      11503.0065

243+23.20  ST                       11673.6584      11503.0065

TL=    730.2577       BEARING= S 85^59'59.97" E

250+53.46  PC                       11622.7181      12231.4853

250+53.46  PC                       11622.7181      12231.4853

LC=    663.2249            CD= 37^59'59.96"

RC=   1000.0000            DC=  5^43'46.48"

257+16.68  PT                       11791.2444      12860.4345

257+16.68  PT                       11791.2444      12860.4345

TL=    362.3508       BEARING= N 56^00'00.07" E

260+79.03  PC                       11993.8683      13160.8370

260+79.03  PC                       11993.8683      13160.8370

LC=   2033.6285            CD= 15^15'07.95"

RC=   7639.4400            DC=  0^45'00.00"

281+12.66  PT                       13340.7547      14676.4813

Horz Alignment	-  continued

The following is an example of the horizontal alignment report using the HORIZ_ALIGN_B_80 template.

Type:	Horz Alignment	Library:	geo.dba	Template:	HORIZ_ALIGN_B_80

April 18, 1994                                            PAGE   1 OF   1

Bearing                 Distance          Northing        Easting

Align:  new

Description:  New alignment US_old

POB     10983.8800      10397.6000

N 24^06'37.11" E             40.4745

TS      11020.8235      10414.1336

TS      11020.8235      10414.1336

ls:    150.0000               Theta:   4^17'49.86"

lt:    100.0295                  st:         50.0268

xs:    149.9156                  ys:          3.7485

SC      11156.1294      10478.7948

SC      11156.1294      10478.7948

Delta: 61^17'43.20"

Radius:   1000.0000

Arc:   1069.8054

Chord:   1019.5146   59^03'18.57"

Tangent:    592.5218

298^24'26.97"      = Radial In

Radius Point:               CC      10680.3901      11358.3812

359^42'10.17"      = Radial Out

CS      11680.3767      11353.1945

CS      11680.3767      11353.1945

ls:    150.0000               Theta:   4^17'49.86"

lt:    100.0295                  st:         50.0268

xs:    149.9156                  ys:          3.7485

ST      11673.6584      11503.0065

ST      11673.6584      11503.0065

S 85^59'59.97" E            730.2577

PC      11622.7181      12231.4853

PC      11622.7181      12231.4853

Delta: 37^59'59.96"

Radius:   1000.0000

Arc:    663.2249

Chord:    651.1361   75^00'00.05"

Tangent:    344.3275

184^00'00.03"      = Radial In

Radius Point:               CC      12620.2822      12301.2419

146^00'00.07"      = Radial Out

PT      11791.2444      12860.4345

PT      11791.2444      12860.4345

N 56^00'00.07" E            362.3508

PC      11993.8683      13160.8370

Vert Alignment	-  Reports on the active vertical alignment of the horizontal alignment specified in the Include Alg field.  The vertical data reported on is derived from the settings in selection criteria which include name, preference, description and fence.

Type:	Vert Alignment	Library:	geo.dba	Template:	VERT_ALIGNMENT

VERTICAL ALIGNMENT LIST

April 18, 1994                                  Page   1 of   1

Project: lab

Alignment: vert2

Description: vert for ha2

POB            STN=0+00.00                 ELEV=  950.00

GRADE=  2.00%               LENGTH=  300.00

PVC            STN=3+00.00                 ELEV=  956.00

PVC            STN=3+00.00                 ELEV=  956.00

ENTRANCE GRADE=  2.00%

LENGTH OF CURVE=  200.00         MID ORDINATE= -2.80

k value= -17.86       Rate of change= -5.6000

STN=3+35.71                 ELEV=  956.36 Sag/Crest Pt

PVT            STN=5+00.00                 ELEV=  948.80

EXIT GRADE= -9.20%

PVT            STN=5+00.00                 ELEV=  948.80

GRADE= -9.20%               LENGTH=  200.00

PVC            STN=7+00.00                 ELEV=  930.40

PVC            STN=7+00.00                 ELEV=  930.40

ENTRANCE GRADE= -9.20%

LENGTH OF CURVE=  400.00        MID ORDINATE=  5.71

k value=  35.01       Rate of change=  2.8560

STN=10+22.13                ELEV=  915.58 Sag/Crest Pt

PVT            STN=11+00.00                ELEV=  916.45

EXIT GRADE=  2.22%

PVT            STN=11+00.00                ELEV=  916.45

GRADE=  2.22%               LENGTH=  384.50

POE            STN=14+84.50                ELEV=  925.00

NOTE:	The Sag/Crest station and elevation data is not correctly reported in this report.  The correct values can be obtained from the Utilities > Review Align command.  A Trouble Report (TL#19277) has been logged.

Track Elements	-  Reports on the track elements in the horizontal alignment specified in the Include Alg field.  Note:  There are no templates available for formatting this report at this time.

Track Turnouts	-  Reports on the track turnouts in the horizontal alignment specified in the Include Alg field.  Note:  There are no templates available for formatting this report at this time.

Table 2 - Report Associations

for rwy.dba, vol.dba and xsec.dba

| Type | .dba library | .dba template | -p options |
| --- | --- | --- | --- |
| Other | rwy.dba | COMMA_RWY_MODELER | none |
|  |  | FINISHED_TEMPLATE | layer_name ls lep rep rs |
|  |  | FINISH_TEMPL_80 | layer_name ls lep rep rs |
|  |  | PIPED_RWY_MODELER | none |
|  |  | PROFILE_PLOT | layer_name tc_name tc_name |
|  |  | ROADWAY_MODELER_XYZ | none |
|  |  | STAKING | layer_name |
|  |  | TEMPL_MULTI_H_80 | -p optional (tc of el dz sl) |
|  |  | TEMPL_MULTI_V_132 | none |
|  |  | TEMPL_MULTI_V_80 | " |
|  |  | TEMPL_SNGL_V_132 | layer_name |
|  |  | TEMPL_SNGL_V_80 | layer_name |
| Other | vol.dba | BASIC_END_AREA | none |
|  |  | COMMA_END_AREA | " |
|  |  | END_AREA_TOPSOIL_MD2 | " |
|  |  | END_AREA_TOPSOIL_MDC | " |
|  |  | FORCED_BALANCE | " |
|  |  | MULTI_MAT_VOL | " |
|  |  | PIPED_END_AREA | " |
| Other | xsec.dba | COMMA_CROSS_SECTION | none |
|  |  | PIPED_CROSS_SECTION | " |
|  |  | X_SECT_MULTI | " |
|  |  | X_SECT_PORT | " |
|  |  | X_SECT_SINGLE | layer_name |

Other	-  This report type selection provides the capability to format the binary files created with the Roadway Modeler, End-Area Volume and Cross-Section commands.  These three commands have the option under Toggles on each of the respective dialog boxes to create binary report files.  The binary report files are then formatted into ASCII reports based on the DB Access template chosen from the appropriate DB Access libray file.  The following DB Access libraries are delivered with the product:

rwy.dba	-	Roadway Modeler

vol.dba	-	End-Area Volume

xsec.dba	-	Cross-Sections

Report Definitions and Examples for rwy.dba

All of the following reports are based on the binary file created when the Roadway Modeler command is processed.  If the binary file was not created, Roadway Modeler will have to be executed again with the Generate DB Access option toggled on.  Following the name of the template is a list of the -p options that are required input for that report template.

COMMA_RWY_MODELER   none

PIPED_RWY_MODELER   none

These reports place all the data available in the binary file into an ASCII file.  The values are delimited with a comma ( , ) or the pipe ( | ) character.

PROFILE_PLOT    layer_name  tc_name  tc_name

The results of this report include the station and elevation of the centerline of the existing and design surface, and the elevation and slope of the specified template control points along the alignment.  The required -p options are the name (layer_name) of the surface to be reported on and two template control (tc_name) names from the template.  The template control names used in the left side of the template design need to be different from the template control names on the right side.  If the names are the same, you cannot be sure which side of the template is being reported on.

PROFILE PLOT LIST

April 18, 1994          DESIGN TEMPLATE LAYER - prop           PAGE   1 OF   1

ROADWAY: new

CENTERLINE  CENTERLINE     lpt2      lpt2         rpt2      rpt2

EXIST ELEV  DESIGN ELEV    ELEV      SLOPE        ELEV      SLOPE

STATION       (FT)        (FT)        (FT)       (%)         (FT)       (%)

229+12.92      960.52     961.00      960.45     0.00        960.16     0.00

229+50.00      961.70     962.02      961.47     2.74        961.18     2.74

229+53.39      961.81     962.11      961.56     2.74        961.27     2.74

230+00.00      963.32     963.39      962.84     2.74        962.55     2.74

230+50.00      964.94     964.76      964.21     2.74        963.92     2.74

231+00.00      966.53     966.13      965.58     2.74        965.29     2.74

231+03.39      966.64     966.22      965.67     2.74        965.38     2.74

231+50.00      968.04     967.50      966.95     2.74        966.66     2.74

232+00.00      968.30     968.87      968.32     2.74        968.03     2.74

232+50.00      967.84     970.16      969.61     2.59        969.32     2.59

233+00.00      969.00     971.18      970.63     2.04        970.34     2.04

233+50.00      970.00     971.92      971.37     1.47        971.08     1.47

234+00.00      970.00     972.37      971.82     0.90        971.53     0.90

234+50.00      970.00     972.53      971.98     0.33        971.69     0.33

235+00.00      970.00     972.41      971.86    -0.24        971.57    -0.24

235+50.00      970.00     972.00      971.45    -0.82        971.16    -0.82

236+00.00      970.00     971.31      970.76    -1.39        970.47    -1.39

236+50.00      970.00     970.33      969.78    -1.96        969.49    -1.96

237+00.00      970.00     969.06      968.51    -2.53        968.22    -2.53

237+50.00      970.00     967.51      966.96    -3.10        966.67    -3.10

238+00.00      970.00     965.67      965.12    -3.68        964.83    -3.68

238+50.00      970.00     963.62      963.07    -4.10        962.78    -4.10

239+00.00      966.70     961.56      961.01    -4.12        960.72    -4.12

239+50.00      962.25     959.50      958.95    -4.12        958.66    -4.12

240+00.00      959.34     957.44      956.89    -4.12        956.60    -4.12

ROADWAY_MODELER_XYZ   none

The results of this report include the horizontal and vertical alignment names, template control names, offset from centerline, coordinates, original ground elevation and cut/fill height.  There are no -p options for this report template.

Roadway Modeler W/xyz

April 18, 1994      01:34 PM                                Page   1 of  15

Layer Name: prop

HA: new

VA: vert

Station               TC       Offset      X coord       Y coord      Z coord

229+12.92            lcut     -26.6819    10373.2458    10994.7794    964.3050

Orig. El.:    964.3050

Cut-Fill Ht.:      0.0000

229+12.92            lpt2     -15.0000    10383.9086    10990.0074    960.4500

Orig. El.:    961.0366

Cut-Fill Ht.:     -0.5866

229+12.92            lpt1     -10.0000    10388.4724    10987.9649    960.7000

Orig. El.:    960.3306

Cut-Fill Ht.:      0.3694

229+12.92                       0.0000    10397.6000    10983.8800    961.0000

Orig. El.:    960.5184

Cut-Fill Ht.:      0.4816

229+12.92                       0.0000    10397.6000    10983.8800    961.0000

Orig. El.:    960.5184

Cut-Fill Ht.:      0.4816

229+12.92            rpt1      12.0000    10408.5531    10978.9781    960.5200

Orig. El.:    960.3415

Cut-Fill Ht.:      0.1785

229+12.92            rpt2      18.0000    10414.0297    10976.5271    960.1600

Orig. El.:    961.3905

Cut-Fill Ht.:     -1.2305

229+12.92            rcut      33.4678    10428.1481    10970.2086    964.0269

Orig. El.:    964.0269

Cut-Fill Ht.:      0.0000

229+50.00            lcut     -25.7091    10389.2807    11028.2272    965.0000

Orig. El.:    965.0000

Cut-Fill Ht.:      0.0000

229+50.00            lpt2     -15.0000    10399.0556    11023.8526    961.4660

Orig. El.:    962.4545

Cut-Fill Ht.:     -0.9885

229+50.00            lpt1     -10.0000    10403.6194    11021.8101    961.7160

Orig. El.:    961.5153

Cut-Fill Ht.:      0.2007

229+50.00                       0.0000    10412.7470    11017.7252    962.0160

Orig. El.:    961.7037

Cut-Fill Ht.:      0.3123

STAKING   layer_name

This report provides the slope staking data for each station.  The slopes, cut/fill values and elevations are included in the report.  The required -p option is the name (layer_name) of the surface to be reported on.

April 18, 1994                                           PAGE   1 OF   7

SLOPE STAKING REPORT

DESIGN TEMPLATE LAYER - prop

SS    CENTERLINE    SS

SLOPE      C/F   OFFSET     ELEV     ELEV     ELEV  OFFSET    C/F    SLOPE

229+12.92

33.00   3.86C  -26.68    964.31   961.00   964.03  33.47   3.87C    25.00

229+50.00

33.00   3.53C  -25.71    965.00   962.02   964.94  33.05   3.76C    25.00

229+53.39

33.00   3.44C  -25.43    965.00   962.11   965.00  32.92   3.73C    25.00

230+00.00

33.00   2.16C  -21.56    965.00   963.39   965.00  27.82   2.45C    25.00

230+50.00

33.00   0.79C  -17.41    965.00   964.76   965.00  22.34   1.08C    25.00

231+00.00

33.00   0.47C  -16.44    966.05   966.13   965.46  18.68   0.17C    25.00

231+03.39

33.00   0.51C  -16.55    966.18   966.22   965.49  18.44   0.11C    25.00

231+50.00

33.00   1.09C  -18.30    968.04   967.50   965.88  20.59   0.78F   -30.00

232+00.00

33.00   1.22C  -18.69    969.54   968.87   966.68  22.47   1.34F   -30.00

232+50.00

33.00   1.23C  -18.72    970.84   970.16   967.71  23.36   1.61F   -30.00

233+00.00

-25.00   1.30F  -20.20    969.33   971.18   968.97  22.57   1.37F   -30.00

FINISHED_TEMPLATE    layer_name  ls  lep  rep  rs

FINISH_TEMPL_80           layer_name  ls  lep  rep  rs

The results of this report include the elevation, offset and slopes for specified template points.  The required -p options are the name (layer_name) of the surface to be reported on and four template control point names as follows: left shoulder (ls), left edge of pavement (lep), right edge of pavement (rep), and right shoulder (rs).

DESIGN TEMPLATE LAYER NAME prop

Horz. Alg.: new                                             PAGE  1  OF  2

Vert. Alg.: vert                                          April 18, 1994

STATION   LT SHLDR       LT EOP          CL         RT EOP      RT SHLDR

229+12.92    960.45        960.70        961.00       960.52       960.16

-15.00        -10.00          0.00        12.00        18.00

Percent Slope: 5.0000        3.0000       -4.0000       -6.0000

229+50.00    961.47        961.72        962.02       961.54       961.18

-15.00        -10.00          0.00        12.00        18.00

Percent Slope: 5.0000        3.0000       -4.0000       -6.0000

229+53.39    961.56        961.81        962.11       961.63       961.27

-15.00        -10.00          0.00        12.00        18.00

Percent Slope: 5.0000        3.0000       -4.0000       -6.0000

230+00.00    962.84        963.09        963.39       962.91       962.55

-15.00        -10.00          0.00        12.00        18.00

Percent Slope: 5.0000        3.0000       -4.0000       -6.0000

230+50.00    964.21        964.46        964.76       964.28       963.92

-15.00        -10.00          0.00        12.00        18.00

Percent Slope: 5.0000        3.0000       -4.0000       -6.0000

231+00.00    965.58        965.83        966.13       965.65       965.29

-15.00        -10.00          0.00        12.00        18.00

Percent Slope: 5.0000        3.0000       -4.0000       -6.0000

231+03.39    965.67        965.92        966.22       965.74       965.38

-15.00        -10.00          0.00        12.00        18.00

Percent Slope: 5.0000        3.0000       -4.0000       -6.0000

231+50.00    966.95        967.20        967.50       967.02       966.66

-15.00        -10.00          0.00        12.00        18.00

Percent Slope: 5.0000        3.0000       -4.0000       -6.0000

232+00.00    968.32        968.57        968.87       968.39       968.03

-15.00        -10.00          0.00        12.00        18.00

Percent Slope: 5.0000        3.0000       -4.0000       -6.0000

232+50.00    969.61        969.86        970.16       969.68       969.32

-15.00        -10.00          0.00        12.00        18.00

Percent Slope: 5.0000        3.0000       -4.0000       -6.0000

233+00.00    970.63        970.88        971.18       970.70       970.34

-15.00        -10.00          0.00        12.00        18.00

Percent Slope: 5.0000        3.0000       -4.0000       -6.0000

TEMPL_MULTI_H_80   tc  of  el  dz  sl

The results of this report can include the template control name, offset distance from centerline, elevation, change in elevation and slope for each template point.  In this report, the -p options are optional and control what data is placed in the report.  The following -p options are available to specify: template control (tc) name, the offset (of) distance, elevation (el), change in elevation (dz), and slope (sl).  If no -p options are given, then all of the data is reported on.

Multiple Templates

April 18, 1994           (ALL TEMPLATE POINTS)     Page   1 of   4

229+12.92      Layer:       prop

Tc:    lcut    lpt2    lpt1                    rpt1    rpt2    rcut

Of:  -26.68  -15.00  -10.00    0.00    0.00   12.00   18.00   33.47

El:  964.31  960.45  960.70  961.00  961.00  960.52  960.16  964.03

Dz:        3.86   -0.25   -0.30    0.00   -0.48   -0.36    3.87

Sl:       33.00   -5.00   -3.00    0.00   -4.00   -6.00   25.00

229+50.00      Layer:       prop

Tc:    lcut    lpt2    lpt1                    rpt1    rpt2    rcut

Of:  -25.71  -15.00  -10.00    0.00    0.00   12.00   18.00   33.05

El:  965.00  961.47  961.72  962.02  962.02  961.54  961.18  964.94

Dz:        3.53   -0.25   -0.30    0.00   -0.48   -0.36    3.76

Sl:       33.00   -5.00   -3.00    0.00   -4.00   -6.00   25.00

229+53.39      Layer:       prop

Tc:    lcut    lpt2    lpt1                    rpt1    rpt2    rcut

Of:  -25.43  -15.00  -10.00    0.00    0.00   12.00   18.00   32.92

El:  965.00  961.56  961.81  962.11  962.11  961.63  961.27  965.00

Dz:        3.44   -0.25   -0.30    0.00   -0.48   -0.36    3.73

Sl:       33.00   -5.00   -3.00    0.00   -4.00   -6.00   25.00

230+00.00      Layer:       prop

Tc:    lcut    lpt2    lpt1                    rpt1    rpt2    rcut

Of:  -21.56  -15.00  -10.00    0.00    0.00   12.00   18.00   27.82

El:  965.00  962.84  963.09  963.39  963.39  962.91  962.55  965.00

Dz:        2.16   -0.25   -0.30    0.00   -0.48   -0.36    2.45

Sl:       33.00   -5.00   -3.00    0.00   -4.00   -6.00   25.00

230+50.00      Layer:       prop

Tc:    lcut    lpt2    lpt1                    rpt1    rpt2    rcut

Of:  -17.41  -15.00  -10.00    0.00    0.00   12.00   18.00   22.34

El:  965.00  964.21  964.46  964.76  964.76  964.28  963.92  965.00

Dz:        0.79   -0.25   -0.30    0.00   -0.48   -0.36    1.08

Sl:       33.00   -5.00   -3.00    0.00   -4.00   -6.00   25.00

231+00.00      Layer:       prop

Tc:    lcut    lpt2    lpt1                    rpt1    rpt2    rcut

Of:  -16.44  -15.00  -10.00    0.00    0.00   12.00   18.00   18.68

El:  966.05  965.58  965.83  966.13  966.13  965.65  965.29  965.46

Dz:        0.47   -0.25   -0.30    0.00   -0.48   -0.36    0.17

Sl:       33.00   -5.00   -3.00    0.00   -4.00   -6.00   25.00

231+03.39      Layer:       prop

Tc:    lcut    lpt2    lpt1                    rpt1    rpt2    rcut

Of:  -16.55  -15.00  -10.00    0.00    0.00   12.00   18.00   18.44

El:  966.18  965.67  965.92  966.22  966.22  965.74  965.38  965.49

Dz:        0.51   -0.25   -0.30    0.00   -0.48   -0.36    0.11

Sl:       33.00   -5.00   -3.00    0.00   -4.00   -6.00   25.00

TEMPL_MULTI_V_132  none

TEMPL_MULTI_V_80    none

TEMPL_SNGL_V_132  layer_name

TEMPL_SNGL_V_80    layer_name

The results of these reports include the transition control point names, the elevation and the distance from centerline for each template point.  The 'MULTI' reports require no -p options and will report on all layer_names from the binary file in either an 80 or 132 column format.  The 'SNGL' reports require a -p option of the name (layer_name) of the template layer to be reported on.  There is an option for an 80 or 132 column format.

***  TEMPLATE  ***

Horz. Alg.: new                                       PAGE   1 OF   3

Vert. Alg.: vert                                      April 20, 1994

TEMPLATE POINT LIST FOR prop

STATION     TRANSITION  ELEVATION  OFFSET  TRANSITION   ELEVATION  OFFSET

NUMBER       CONTROL      (FT)      (FT)    CONTROL       (FT)     (FT)

229+12.92         lcut   964.31   -26.68         lpt2   960.45   -15.00

lpt1   960.70   -10.00                961.00     0.00

961.00     0.00         rpt1   960.52    12.00

rpt2   960.16    18.00         rcut   964.03    33.47

229+50.00         lcut   965.00   -25.71         lpt2   961.47   -15.00

lpt1   961.72   -10.00                962.02     0.00

962.02     0.00         rpt1   961.54    12.00

rpt2   961.18    18.00         rcut   964.94    33.05

229+53.39         lcut   965.00   -25.43         lpt2   961.56   -15.00

lpt1   961.81   -10.00                962.11     0.00

962.11     0.00         rpt1   961.63    12.00

rpt2   961.27    18.00         rcut   965.00    32.92

230+00.00         lcut   965.00   -21.56         lpt2   962.84   -15.00

lpt1   963.09   -10.00                963.39     0.00

963.39     0.00         rpt1   962.91    12.00

rpt2   962.55    18.00         rcut   965.00    27.82

230+50.00         lcut   965.00   -17.41         lpt2   964.21   -15.00

lpt1   964.46   -10.00                964.76     0.00

964.76     0.00         rpt1   964.28    12.00

rpt2   963.92    18.00         rcut   965.00    22.34

231+00.00         lcut   966.05   -16.44         lpt2   965.58   -15.00

lpt1   965.83   -10.00                966.13     0.00

966.13     0.00         rpt1   965.65    12.00

rpt2   965.29    18.00         rcut   965.46    18.68

231+03.39         lcut   966.18   -16.55         lpt2   965.67   -15.00

lpt1   965.92   -10.00                966.22     0.00

966.22     0.00         rpt1   965.74    12.00

rpt2   965.38    18.00         rcut   965.49    18.44

231+50.00         lcut   968.04   -18.30         lpt2   966.95   -15.00

lpt1   967.20   -10.00                967.50     0.00

967.50     0.00         rpt1   967.02    12.00

rpt2   966.66    18.00        rfill   965.88    20.59

Report Definitions and Examples for vol.dba

All of the following reports are based on the binary file created when the End-Area volume command is processed.  If the binary file was not created, End-Area volumes will have to be recalculated again with the Generate DB Access option toggled on.  There are no -p options required for any of these reports.

COMMA_END_AREA   none

PIPED_END_AREA   none

These reports place all the data available in the binary file into an ASCII file.  The values are delimited with the comma ( , ) or pipe ( | ) character.

MULTI_MAT_VOL   none

This report contains the area and volume for cut and fill for each material type at each station along the alignment.  There are no -p options required for this report.

BASIC_END_AREA   none

This report contains the area and volume for cut and fill at each station with Page Totals and a Grand Total.  There are no -p options required for this report.

* * * EARTHWORK * * *                                       end_area_vol report

April 18, 1994     04:50 PM                                                                                                        Page   1 of   5

BASELINE   - - - - - - - - - - - - - -   STATION  - - - - - - - - - - - - - -     - - - - - - - -  ADDED QUANTITIES  - - - - - - - -

STATION    - - - - - - CUT - - - - - - - -   - - - - -  FILL  - - - - - - - -     - - - - - CUT - - - - -   - - - - - FILL - - - - -    MASS

NUMBER     FACT   AREA    VOLUME  ADJUSTED   FACT  AREA     VOLUME   ADJUSTED     FACT  VOLUME   ADJUSTED   FACT  VOLUME    ADJUSTED    ORDINATE

(sq ft)   (cu yd)  (cu yd)        (sq ft)   (cu yd)  (cu yd)            (cu yd)  (cu yd)          (cu yd)   (cu yd)     (cu yd)

--------   ---- -------- -------- --------   ---- --------  -------- --------     ---- --------  --------   ----  --------  --------    --------

0          1.00     0.00        0        0   1.00     0.00         0        0     1.00      250       250   1.00         0         0         250

100        1.00    38.82       72       72   1.00   128.07       237      237     1.00      500       500   1.00         0         0         804

200        1.00     0.00       72       72   1.00   306.57       805      805     1.00        0         0   1.00         0         0         815

300        1.00     0.00        0        0   1.00   487.81      1471     1471     1.00     1000      1000   1.00         0         0        1705

400        1.00     0.00        0        0   1.00   639.27      2087     2087     1.00     1000      1000   1.00         0         0        2547

500        1.00     0.00        0        0   1.00   758.59      2589     2589     1.00        0         0   1.00         0         0        2353

581        1.00     0.00        0        0   1.00   825.35      2367     2367     1.00        0         0   1.00         0         0        2174

600        1.00     0.00        0        0   1.00   842.11       597      597     1.00        0         0   1.00         0         0        2130

700        1.00     0.00        0        0   1.00  1118.15      3630     3630     1.00        0         0   1.00         0         0        1856

800        1.00     0.00        0        0   1.00  1267.25      4417     4417     1.00        0         0   1.00         0         0        1524

900        1.00     0.00        0        0   1.00  1202.91      4574     4574     1.00        0         0   1.00         0         0        1179

965        1.00     0.00        0        0   1.00  1224.85      2928     2928     1.00        0         0   1.00         0         0         959

1000       1.00     0.00        0        0   1.00  1220.14      1579     1579     1.00        0         0   1.00         0         0         840

. . .              . . .                                 . . .                      . . .                         . . .

2700       1.00  1487.10     4413     4413   1.00     0.00         0        0     1.00        0         0   1.00         0         0       -2457

2800       1.00  1159.35     4901     4901   1.00     0.00         0        0     1.00        0         0   1.00         0         0        2444

2853       1.00   989.06     2104     2104   1.00     0.00         0        0     1.00        0         0   1.00         0         0        4549

2854       1.00   987.40       32       32   1.00     0.00         0        0     1.00        0         0   1.00         0         0        4580

2900       1.00   896.38     1613     1613   1.00     0.00         0        0     1.00        0         0   1.00         0         0        6193

3000       1.00   877.30     3285     3285   1.00     0.00         0        0     1.00        0         0   1.00         0         0        9478

3100       1.00   924.37     3336     3336   1.00     0.00         0        0     1.00        0         0   1.00         0         0       12814

3122       1.00   880.62      728      728   1.00     0.00         0        0     1.00        0         0   1.00         0         0       13542

3200       1.00   667.42     2242     2242   1.00    37.66        55       55     1.00        0         0   1.00         0         0       15730

================================================================================================================================================

Page Total:       23095    23095                     49661    49661              2750      2750                0         0

================================================================================================================================================

. . .              . . .                                 . . .                      . . .                         . . .

================================================================================================================================================

Grand Total:       34762    34762                     51629    51629              2750      2750                0         0

================================================================================================================================================

END_AREA_TOPSOIL_MD2   none

END_AREA_TOPSOIL_MDC   none

These reports show the amount of stripping based on the parameters set when End-Area volumes were calculated.  The only difference between these two reports is the page formatting.  There are no -p options required for these reports.

* * * EARTHWORK * * *                                                            end_area_vol report

April 18, 1994     04:51 PM                                                                                                                 Page   1 of  36

BASELINE   - - - - - - - - - - - - - -   STATION  - - - - - - - - - - - - - -                      - - - - - - - -  ADDED QUANTITIES  - - - - - - - -

STATION    - - - - - - CUT - - - - - - - -   - - - - -  FILL  - - - - - - - -              - - - - - - CUT - - - - - - - -   - - - - -  FILL  - - - - - - - -     MASS

NUMBER     FACT   AREA    VOLUME  ADJUSTED   FACT  AREA     VOLUME   ADJUSTED    ADDED     FACT           VOLUME  ADJUSTED   FACT           VOLUME   ADJUSTED   ORDINATE

(sq ft)   (cu yd)  (cu yd)        (sq ft)   (cu yd)  (cu yd)                              (cu yd)  (cu yd)                  (cu yd)  (cu yd)    (cu yd)

--------   ---- -------- -------- --------   ---- --------  -------- --------              ----          -------- --------   ----           -------- --------   --------

0          1.00     0.00        0        0   1.00     0.00         0        0    ADDED     1.00               250      250   1.00                  0        0

Topsoil         0                  Topsoil         0

Undercut        0

--------                           --------                                     --------                           --------   --------

Station Total:                    0                                  0                                          250                                  0        250

________________________________________________________________________________________________________________________________________________________________________

100        1.00    38.82       72       72   1.00   128.07       237      237    ADDED     1.00               500      500   1.00                  0        0

Topsoil       -45                  Topsoil       113

Undercut        2

--------                           --------                                     --------                           --------   --------

Station Total:                   29                                351                                          500                                  0        429

________________________________________________________________________________________________________________________________________________________________________

200        1.00     0.00       72       72   1.00   306.57       805      805    ADDED     1.00                 0        0   1.00                  0        0

Topsoil       -45                  Topsoil       253

Undercut        2

--------                           --------                                     --------                           --------   --------

Station Total:                   29                               1058                                            0                                  0       -599

________________________________________________________________________________________________________________________________________________________________________

________________________________________________________________________________________________________________________________________________________________________

========================================================================================================================================================================

Page Total:         144       59                      2513     3185                                   4     1750                         4        0

========================================================================================================================================================================

. . .              . . .                                 . . .                      . . .                         . . .

========================================================================================================================================================================

Grand Total:       34762    30582                     51629    59390                                2750     2750                         0        0

========================================================================================================================================================================

FORCED_BALANCE   none

This report shows the balance factor with the area and volume for cut and fill at each station.  In the following example the balance out station is 19+00.  There are no -p options required for this report.

* * * EARTHWORK * * *                                       end_area_vol report

April 18, 1994     04:52 PM                                                                              Page   1 of   5

BASELINE        - - - - - - - - - - -   STATION  - - - - - - - - - - -    - - - -  ADDED QUANTITIES  - - - -

STATION    BAL  - - - - - - CUT - - - - -   - - - - -  FILL  - - - - -    - - - CUT - - -   - - - FILL - - -     MASS

NUMBER     FACT   AREA    VOLUME  ADJUSTED   AREA    VOLUME   ADJUSTED   VOLUME   ADJUSTED   VOLUME  ADJUSTED  ORDINATE

(sq ft)   (cu yd)  (cu yd)  (sq ft)  (cu yd)  (cu yd)    (cu yd)  (cu yd)    (cu yd) (cu yd)   (cu yd)

--------   ---- -------- -------- --------  -------- -------- --------   -------- --------  -------- --------  --------

0          ****     0.00        0        0      0.00        0        0        250      250         0        0       250

100        ****    38.82       72       72    128.07      237      237        500      500       237        0       804

200        ****     0.00       72       72    306.57      805      805          0        0       805        0       815

300        ****     0.00        0        0    487.81     1471     1471       1000     1000      1471        0      1705

400        ****     0.00        0        0    639.27     2087     2087       1000     1000      2087        0      2547

500        ****     0.00        0        0    758.59     2589     2589          0        0      2589        0      2353

581        ****     0.00        0        0    825.35     2367     2367          0        0      2367        0      2174

600        ****     0.00        0        0    842.11      597      597          0        0       597        0      2130

700        ****     0.00        0        0   1118.15     3630     3630          0        0      3630        0      1856

800        ****     0.00        0        0   1267.25     4417     4417          0        0      4417        0      1524

900        ****     0.00        0        0   1202.91     4574     4574          0        0      4574        0      1179

965        ****     0.00        0        0   1224.85     2928     2928          0        0      2928        0       959

1000       ****     0.00        0        0   1220.14     1579     1579          0        0      1579        0       840

1100       ****     0.00        0        0      0.00     2260     2260          0        0      2260        0       670

1200       ****     0.00        0        0      0.00        0        0          0        0         0        0       670

1300       ****     0.00        0        0      0.00        0        0          0        0         0        0       670

1400       ****     0.00        0        0   1202.95     2228     2228          0        0      2228        0       502

1500       ****     0.00        0        0    846.91     3796     3796          0        0      3796        0       216

1600       ****     0.00        0        0    461.47     2423     2423          0        0      2423        0        34

1688       ****     2.85        5        5    226.42     1117     1117          0        0      1117        0       -45

1700       ****     4.11        2        2    228.76      104      104          0        0       104        0       -52

1800       ****     3.05       13       13     87.52      586      586          0        0       586        0       -82

1900       ****    66.10      128      128    239.44      605      605          0        0       605        0         0

1985       1.00     0.00      104      104    519.77     1189     1189          0        0      1189        0     -1085

2000       1.00     0.00        0        0    543.90      304      304          0        0       304        0     -1390

2100       1.00     0.00        0        0    749.65     2395     2395          0        0      2395        0     -3785

2200       1.00     0.00        0        0    475.50     2269     2269          0        0      2269        0     -6054

2300       1.00    46.09       85       85    302.80     1441     1441          0        0      1441        0     -7410

NOTE:	A Trouble Report (TL#19278) has been logged against this report to increase the field display size for the balance factor.  The balance factor can be found in the comma or piped format reports.

Report Definitions and Examples for xsec.dba

All of the following reports are based on the binary file created when the Cross-Section command is processed.  If the binary file was not created, the cross-sections will have to be displayed again with the Generate DB Access option toggled on.  Following the name of the template is a list of the -p options that are required input for that report template.

COMMA_CROSS_SECTION   none

PIPED_CROSS_SECTION   none

These reports place all the data available in the binary file into an ASCII file.  The values are delimited with the comma ( , ) or pipe ( | ) character.

X_SECT_MULTI   none

X_SECT_SINGLE   layer_name

These reports provide the elevation and offset distances at each station based on the station interval that the cross-sections were plotted at and the major tick interval for each station.  The 'MULTI' template will report on all the layers that were included with the set of cross-sections displayed to the design file.  The 'SINGLE' template reports only on the layer name specified in the -p options list.

PAGE   1 OF   4

***  TERRAIN  ***

April 18, 1994

CROSS-SECTION LIST FOR prop

STATION      ELEV      DIST     ELEV      DIST     ELEV      DIST

NUMBER       (FT)      (FT)     (FT)      (FT)     (FT)      (FT)

263+00    945.48   -25.00    945.03     0.00    942.84    25.00

263+50    946.71     0.00    944.52    25.00

264+00    948.39     0.00    946.21    25.00

264+50    950.04     0.00    947.86    25.00

265+00    951.62     0.00

265+50    953.11     0.00

266+00    954.99   -25.00    954.54     0.00    954.99    25.00

266+50    956.34   -25.00    955.88     0.00    956.34    25.00

267+00    957.61   -25.00    957.15     0.00    957.61    25.00

267+50    958.80   -25.00    958.35     0.00    958.80    25.00

268+00    959.93   -25.00    959.47     0.00    959.93    25.00

268+50    960.97   -25.00    960.52     0.00    960.97    25.00

269+00    961.94   -25.00    961.49     0.00    961.94    25.00

269+50    962.83   -25.00    962.38     0.00

270+00    961.01   -25.00    963.20     0.00    961.01    25.00

270+50    961.76   -25.00    963.94     0.00    961.76    25.00

271+00    962.42   -25.00    964.61     0.00    962.42    25.00

271+50    963.02   -25.00    965.20     0.00    963.02    25.00

272+00    963.53   -25.00    965.72     0.00    963.53    25.00

272+50    966.19     0.00

X_SECT_PORT   none

This report produces a vertical listing of the station, offset, and elevation for each station as defined in the station interval when the cross-sections were displayed to the design file.  There are no -p options required for this report.

April 18, 1994                         Page    1 of  15

CROSS-SECTION LIST FOR  prop

STATION     Offset    Elevation

229+13    -25.0000     963.7500

0.0000     961.0000

25.0000     961.9100

229+50    -25.0000     964.7660

0.0000     962.0160

25.0000     962.9260

229+53    -25.0000     964.8590

0.0000     962.1090

25.0000     963.0190

230+00      0.0000     963.3860

25.0000     964.2960

230+50      0.0000     964.7560

231+00      0.0000     966.1260

231+03      0.0000     966.2190

231+50      0.0000     967.4960

232+00      0.0000     968.8660

232+50      0.0000     970.1619

233+00      0.0000     971.1831

233+50      0.0000     971.9183

234+00      0.0000     972.3677

234+50      0.0000     972.5311

235+00      0.0000     972.4086

235+50      0.0000     972.0002

236+00      0.0000     971.3059

236+50      0.0000     970.3257

Binary File Structure - geo

Report Type             Structure

-----------             -----------

Area                    TDPFTAG_RPT

List                    TDPLIST_RPT

Traverse                TDPTRAV_RPT

Clearance               TDPCLER_RPT

Center Stakeout         TDPCSTK_RPT

Radial Stakeout         TDPRSTK_RPT

Station Single          TDPSOFF_RPT

Station Base            TDPSOFF_RPT

Station Offset          TDPSOFF_RPT

Horizontal Alignment    HAVA_RPT_ALL_4

Vertical Alignment      HAVA_RPT_ALL_4

Annotation Tables       TDPATBL_RPT

**********************************************************

Structure:      TDPFTAG_RPT

Table Name:     tdpftag_rpt

Table No.       1

Table condition:

Col.                                                            Byte

No.     Col. Name       Description     Col. Type       Length  Offset

1       prj                             char            64      0

2       alg                             char            64      64

3       area_0                          8-byte float    8       128

4       area_1                          8-byte float    8       136

5       area_2                          8-byte float    8       144

6       area_3                          8-byte float    8       152

7       cgf                             8-byte float    8       160

8       msg                             char            40      168

208

***********************************************************************

Structure:      TDPLIST_RPT

Table Name:     tdplist_rpt

Table No.       1

Table condition:

Col.                                                            Byte

No.     Col. Name       Description     Col. Type       Length  Offset

1       prj                             char            64      0

2       alg                             char            64      64

3       nam                             char            24      128

4       x                               8-byte float    8       152

5       y                               8-byte float    8       160

6       z                               8-byte float    8       168

7       des                             char            40      176

8       ftr                             char            32      216

9       ptype                           char            8       248

10      msg                             char            40      256

296

************************************************************************

Structure:      TDPTRAV_RPT

Table Name:     tdptrav_rpt

Table No.       1

Table condition:

Col.                                    Byte

No.     Col. Name       Description     Col. Type       Length  Offset

1       prj                             char            64      0

2       alg                             char            64      64

3       bs_nam                          char            24      128

4       bs_x                            8-byte float    8       152

5       bs_y                            8-byte float    8       160

6       bs_z                            8-byte float    8       168

7       bs_des                          char            40      176

8       bs_ftr                          char            32      216

9       oc_nam                          char            24      248

10      oc_x                            8-byte float    8       272

11      oc_y                            8-byte float    8       280

12      oc_z                            8-byte float    8       288

13      oc_des                          char            40      296

14      oc_ftr                          char            32      336

15      fs_nam                          char            24      368

16      fs_x                            8-byte float    8       392

17      fs_y                            8-byte float    8       400

18      fs_z                            8-byte float    8       408

19      fs_des                          char            40      416

20      fs_ftr                          char            32      456

21      dir                             char            24      488

22      dfl                             char            24      512

23      al                              char            24      536

24      ar                              char            24      560

25      dst                             8-byte float    8       584

26      grd                             8-byte float    8       592

27      cgf                             8-byte float    8       600

28      dx                              8-byte float    8       608

29      dy                              8-byte float    8       616

30      dz                              8-byte float    8       624

31      msg                             char            40      632

672

************************************************************************

Structure:      TDPCLER_RPT

Table Name:             tdpcler_rpt

Table No.               1

Table condition:

Col.                                                            Byte

No.     Col. Name       Description     Col. Type       Length  Offset

1       prj                             char            64      0

2       hcl                             char            64      64

3       vcl                             char            64      128

4       hol                             char            64      192

5       op_nam          offset pt name  char            24      256

6       op_x                            8-byte float    8       280

7       op_y                            8-byte float    8       288

8       op_z                            8-byte float    8       296

9       op_des                          char            40      304

10      op_ftr                          char            32      344

11      cl_nam                          char            24      376

12      cl_x                            8-byte float    8       400

13      cl_y                            8-byte float    8       408

14      cl_z                            8-byte float    8       416

15      cl_stn                          char            32      424

16      offset                          8-byte float    8       456

17      taz             tangential az   char            24      464

18      raz             radial az       char            24      488

19      msg                             char            40      512

552

***********************************************************************

Structure:      TDPCSTK_RPT

Table Name:     tdpcstkt_rpt

Table No.       1

Table condition:

Col.                                                            Byte

No.     Col. Name       Description     Col. Type       Length  Offset

1       prj                             char            64      0

2       hcl                             char            64      64

3       hol                             char            64      128

4       vol                             char            64      192

5       bs_nam                          char            24      256

6       bs_x                            8-byte float    8       280

7       bs_y                            8-byte float    8       288

8       bs_z                            8-byte float    8       296

9       bs_des                          char            40      304

10      bs_ftr                          char            32      344

11      oc_nam                          char            24      376

12      oc_x                            8-byte float    8       400

13      oc_y                            8-byte float    8       408

14      oc_z                            8-byte float    8       416

15      oc_des                          char            40      424

16      oc_ftr                          char            32      464

17      fs_nam                          char            24      496

18      fs_x                            8-byte float    8       520

19      fs_y                            8-byte float    8       528

20      fs_z                            8-byte float    8       536

21      fs_stn                          char            32      544

22      offset                          8-byte float    8       576

23      dir                             char            24      584

24      dfl                             char            24      608

25      al                              char            24      632

26      ar                              char            24      656

27      dst                             8-byte float    8       680

28      grd                             8-byte float    8       688

29      cgf                             8-byte float    8       696

30      msg                             char            40      704

744

************************************************************************

Structure:      TDPRSTK_RPT

Table Name:     tdprstk_rpt

Table No.       1

Table condition:

Col.                                                            Byte

No.     Col. Name       Description     Col. Type       Length  Offset

1       prj                             char            64      0

2       alg_1                           char            64      64

3       alg_2                           char            64      128

4       bs_nam                          char            24      192

5       bs_x                            8-byte float    8       216

6       bs_y                            8-byte float    8       224

7       bs_z                            8-byte float    8       232

8       bs_des                          char            40      240

9       bs_ftr                          char            32      280

10      oc_nam                          char            24      312

11      oc_x                            8-byte float    8       336

12      oc_y                            8-byte float    8       344

13      oc_z                            8-byte float    8       352

14      oc_des                          char            40      360

15      oc_ftr                          char            32      400

16      fs_nam                          char            24      432

17      fs_x                            8-byte float    8       456

18      fs_y                            8-byte float    8       464

19      fs_z                            8-byte float    8       472

20      fs_des                          char            40      480

21      fs_ftr                          char            32      520

22      dir                             char            24      552

23      dfl                             char            24      576

24      al                              char            24      600

25      ar                              char            24      624

26      dst                             8-byte float    8       648

27      grd                             8-byte float    8       656

28      cgf                             8-byte float    8       664

29      msg                             char            40      672

712

**********************************************************************

Structure:      TDPSOFF_RPT

Table Name:     tdpsoff_rpt

Table No.       1

Table condition:

Col.                                                            Byte

No.     Col. Name       Description     Col. Type       Length  Offset

1       prj                             char            64      0

2       hcl                             char            64      64

3       vcl                             char            64      128

4       hol                             char            64      192

5       vol                             char            64      256

6       cl_nam                          char            24      320

7       cl_x                            8-byte float    8       344

8       cl_y                            8-byte float    8       352

9       cl_z                            8-byte float    8       360

10      cl_stn                          char            32      368

11      ol_nam                          char            24      400

12      ol_x                            8-byte float    8       424

13      ol_y                            8-byte float    8       432

14      ol_z                            8-byte float    8       440

15      ol_stn                          char            32      448

16      op_x                            8-byte float    8       480

17      op_y                            8-byte float    8       488

18      offset_2                        8-byte float    8       496

19      offset_1                        8-byte float    8       504

20      grade_1                         8-byte float    8       512

21      grade_2                         8-byte float    8       520

22      clraz           cl radial az    char            24      528

23      cltaz           cl tan az       char            24      552

24      olraz           ol radial az    char            24      576

25      oltaz           ol tan az       char            24      600

26      msg                             char            40      624

664

************************************************************************

Structure:      HAVA_RPT_ALL_4

Table Name:             phdr

Table No.               1

Table condition:        0

Col.                                                            Byte

No.     Col. Name       Description     Col. Type       Length  Offset

1       type                            4-byte Int      4       0

2       unused                          char            4       4

3       prj                             char            64      8

4       alg                             char            64      72

5       des                             char            40      136

6       junk                            char            200     176

7       junk2                           char            200     376

8       junk3                           char            64      576

640

Table Name:             h_ele_point

Table No.               2

Table condition:        512

Col.                                                            Byte

No.     Col. Name       Description     Col. Type       Length  Offset

1       type                            4-byte Int      4       0

2       unused                          4-byte Int      4       4

3       csid                            4-byte Int      4       8

4       unused1                         4-byte Int      4       12

5       pi_name                         char            24      16

6       pi_type                         char            8       40

7       pi_stn                          char            32      48

8       pi_ftr                          char            32      80

9       pi_x                            8-byte float    8       112

10      pi_y                            8-byte float    8       120

11      pi_z                            8-byte float    8       128

12      ts1                             8-byte float    8       136

13      ts2                             8-byte float    8       144

14      sts2                            8-byte float    8       152

15      sts2                            8-byte float    8       160

16      tstaz                           char            24      168

17      sttaz                           char            24      192

18      delta                           char            24      216

19      junk                            char            200     240

20      junk2                           char            200     440

640

Table Name:             h_ele_line

Table No.               3

Table condition:        1

Col.                                                            Byte

No.     Col. Name       Description     Col. Type       Length  Offset

1       type                            4-byte Int      4       0

2       unused                          4-byte Int      4       4

3       csid                            4-byte Int      4       8

4       unused1                         4-byte Int      4       12

5       p1_name                         char            24      16

6       p1_type                         char            8       40

7       p1_stn                          char            32      48

8       p1_ftr                          char            32      80

9       p1_x                            8-byte float    8       112

10      p1_y                            8-byte float    8       120

11      p1_z                            8-byte float    8       128

12      p2_name                         char            24      136

13      p2_type                         char            8       160

14      p2_stn                          char            32      168

15      p2_ftr                          char            32      200

16      p2_x                            8-byte float    8       232

17      p2_y                            8-byte float    8       240

18      p2_z                            8-byte float    8       248

19      len                             8-byte float    8       256

20      tangential                      char            24      264

21      ftr                             char            32      288

22      junk                            char            200     320

23      junk2                           char            120     520

640

Table Name:             h_ele_circle

Table No.               4

Table condition:        2

Col.                                                            Byte

No.     Col. Name       Description     Col. Type       Length  Offset

1       type                            4-byte Int      4       0

2       unused                          4-byte Int      4       4

3       csid                            4-byte Int      4       8

4       unused1                         4-byte Int      4       12

5       pc_name                         char            24      16

6       pc_type                         char            8       40

7       pc_stn                          char            32      48

8       pc_ftr                          char            32      80

9       pc_x                            8-byte float    8       112

10      pc_y                            8-byte float    8       120

11      pc_z                            8-byte float    8       128

12      cc_name                         char            24      136

13      cc_type                         char            8       160

14      cc_stn                          char            32      168

15      cc_ftr                          char            32      200

16      cc_x                            8-byte float    8       232

17      cc_y                            8-byte float    8       240

18      cc_z                            8-byte float    8       248

19      pt_name                         char            24      256

20      pt_type                         char            8       280

21      pt_stn                          char            32      288

22      pt_ftr                          char            32      320

23      pt_x                            8-byte float    8       352

24      pt_y                            8-byte float    8       360

25      pt_z                            8-byte float    8       368

26      radius                          8-byte float    8       376

27      arc                             8-byte float    8       384

28      chd                             8-byte float    8       392

29      tc                              8-byte float    8       400

30      mo                              8-byte float    8       408

31      ext                             8-byte float    8       416

32      delta                           char            24      424

33      doc                             char            24      448

34      pcraz                           char            24      472

35      pctaz                           char            24      496

36      ptraz                           char            24      520

37      pttaz                           char            24      544

38      chdaz                           char            24      568

39      ftr                             char            32      592

40      junk                            char            16      624

640

Table Name:             h_ele_spiral

Table No.               5

Table condition:        4

Col.                                                            Byte

No.     Col. Name       Description     Col. Type       Length  Offset

1       type                            4-byte Int      4       0

2       unused                          4-byte Int      4       4

3       csid                            4-byte Int      4       8

4       unused1                         4-byte Int      4       12

5       bs_name                         char            24      16

6       bs_type                         char            8       40

7       bs_stn                          char            32      48

8       bs_ftr                          char            32      80

9       bs_x                            8-byte float    8       112

10      bs_y                            8-byte float    8       120

11      bs_z                            8-byte float    8       128

12      pi_name                         char            24      136

13      pi_type                         char            8       160

14      pi_stn                          char            32      168

15      pi_ftr                          char            32      200

16      pi_x                            8-byte float    8       232

17      pi_y                            8-byte float    8       240

18      pi_z                            8-byte float    8       248

19      es_name                         char            24      256

20      es_type                         char            8       280

21      es_stn                          char            32      288

22      es_ftr                          char            32      320

23      es_x                            8-byte float    8       352

24      es_y                            8-byte float    8       360

25      es_z                            8-byte float    8       368

26      ent_r                           8-byte float    8       376

27      ext_r                           8-byte float    8       384

28      ls                              8-byte float    8       392

29      ks                              8-byte float    8       400

30      p                               8-byte float    8       408

31      ys                              8-byte float    8       416

32      xs                              8-byte float    8       424

33      lc                              8-byte float    8       432

34      st                              8-byte float    8       440

35      lt                              8-byte float    8       448

36      a                               8-byte float    8       456

37      theta                           char            24      464

38      raz1                            char            24      488

39      raz2                            char            24      512

40      taz1                            char            24      536

41      taz2                            char            24      560

42      caz                             char            24      584

43      ftr                             char            32      608

640

Table Name:             h_ele_eqn

Table No.               6

Table condition:        1024

Col.                                                            Byte

No.     Col. Name       Description     Col. Type       Length  Offset

1       type                            4-byte Int      4       0

2       unused                          4-byte Int      4       4

3       csid                            4-byte Int      4       8

4       id                              4-byte Int      4       12

5       bk                              char            32      16

6       ahd                             char            32      48

7       junk                            char            200     80

8       junk2                           char            200     280

9       junk3                           char            160     480

640

Table Name:             v_ele_line

Table No.               7

Table condition:        8

Col.                                                            Byte

No.     Col. Name       Description     Col. Type       Length  Offset

1       type                            4-byte Int      4       0

2       unused                          4-byte Int      4       4

3       csid                            4-byte Int      4       8

4       unused1                         4-byte Int      4       12

5       pvi1_type                       char            8       16

6       pvi1_stn                        char            32      24

7       pvi1_ftr                        char            32      56

8       pvi1_elev                       8-byte float    8       88

9       pvi2_type                       char            8       96

10      pvi2_stn                        char            32      104

11      pvi2_ftr                        char            32      136

12      pvi2_elev                       8-byte float    8       168

13      len                             8-byte float    8       176

14      grade                           8-byte float    8       184

15      ftr                             char            32      192

16      junk                            char            200     224

17      junk2                           char            200     424

18      junk3                           char            16      624

640

Table Name:             v_ele_circle

Table No.               8

Table condition:        16

Col.                                    Byte

No.     Col. Name       Description     Col. Type       Length  Offset

1       type                            4-byte Int      4       0

2       unused                          4-byte Int      4       4

3       csid                            4-byte Int      4       8

4       unused1                         4-byte Int      4       12

5       pvc_type                        char            8       16

6       pvc_stn                         char            32      24

7       pvc_ftr                         char            32      56

8       pvc_elev                        8-byte float    8       88

9       pvi_type                        char            8       96

10      pvi_stn                         char            32      104

11      pvi_ftr                         char            32      136

12      pvi_elev                        8-byte float    8       168

13      pvt_type                        char            8       176

14      pvt_stn                         char            32      184

15      pvt_ftr                         char            32      216

16      pvt_elev                        8-byte float    8       248

17      cc_type                         char            8       256

18      cc_stn                          char            32      264

19      cc_ftr                          char            32      296

20      cc_elev                         8-byte float    8       328

21      tp_type                         char            8       336

22      tp_stn                          char            32      344

23      tp_ftr                          char            32      376

24      tp_elev                         8-byte float    8       408

25      radius                          8-byte float    8       416

26      grade_1                         8-byte float    8       424

27      grade_2                         8-byte float    8       432

28      ftr                             char            32      440

29      junk                            char            168     472

640

Table Name:             v_ele_parabola

Table No.               9

Table condition:        32

Col.                                                            Byte

No.     Col. Name       Description     Col. Type       Length  Offset

1       type                            4-byte Int      4       0

2       unused                          4-byte Int      4       4

3       csid                            4-byte Int      4       8

4       unsymetrical                    4-byte Int      4       12

5       pvc_type                        char            8       16

6       pvc_stn                         char            32      24

7       pvc_ftr                         char            32      56

8       pvc_elev                        8-byte float    8       88

9       pvi_type                        char            8       96

10      pvi_stn                         char            32      104

11      pvi_ftr                         char            32      136

12      pvi_elev                        8-byte float    8       168

13      pvt_type                        char            8       176

14      pvt_stn                         char            32      184

15      pvt_ftr                         char            32      216

16      pvt_elev                        8-byte float    8       248

17      tp_type                         char            8       256

18      tp_stn                          char            32      264

19      tp_ftr                          char            32      296

20      tp_elev                         8-byte float    8       328

21      lvc1                            8-byte float    8       336

22      lvc2                            8-byte float    8       344

23      mo                              8-byte float    8       352

24      grade1                          8-byte float    8       360

25      grade2                          8-byte float    8       368

26      rate_of_change1                 8-byte float    8       376

27      rate_of_change1                 8-byte float    8       384

28      k_value1                        8-byte float    8       392

29      k_value2                        8-byte float    8       400

30      ext                             8-byte float    8       408

31      adif                            8-byte float    8       416

32      ftr                             char            32      424

33      junk                            char            184     456

640

Table Name:             turnout_pt

Table No.               10

Table condition:        4096

Col.                                    Byte

No.     Col. Name       Description     Col. Type       Length  Offset

1       type                            4-byte Int      4       0

2       unused                          4-byte Int      4       4

3       topnt_name                      char            24      8

4       topnt_ftr                       char            32      32

5       azimuth                         char            24      64

6       to_pnt_x                        8-byte float    8       88

7       to_pnt_y                        8-byte float    8       96

8       to_pnt_z                        8-byte float    8       104

9       radbeg                          8-byte float    8       112

10      radend                          8-byte float    8       120

11      junk                            char            200     128

12      junk2                           char            200     328

13      junk3                           char            112     528

640

Table Name:             h_rpt_pt

Table No.               11

Table condition:        128

Col.                                                            Byte

No.     Col. Name       Description     Col. Type       Length  Offset

1       type                            4-byte Int      4       0

2       unused                          4-byte Int      4       4

3       shlim_name                      char            24      8

4       shlim_type                      char            8       32

5       shlim_stn                       char            32      40

6       shlim_ftr                       char            32      72

7       shlim_x                         8-byte float    8       104

8       shlim_y                         8-byte float    8       112

9       shlim_z                         8-byte float    8       120

10      junk                            char            200     128

11      junk2                           char            200     328

12      junk3                           char            112     528

640

Table Name:             ann_h_ele_point

Table No.               1

Table condition:        512

Col.                                                            Byte

No.     Col. Name       Description     Col. Type       Length  Offset

1       type                            4-byte Int      4       0

2       unused                          4-byte Int      4       4

3       p_name                          char            24      8

4       p_des                           char            40      32

5       p_ftr                           char            32      72

6       p_x                             8-byte float    8       104

7       p_y                             8-byte float    8       112

8       p_z                             8-byte float    8       120

9       junk                            char            24      128

152

Table Name:             ann_h_ele_line

Table No.               2

Table condition:        1

Col.                                                            Byte

No.     Col. Name       Description     Col. Type       Length  Offset

1       type                            4-byte Int      4       0

2       unused                          4-byte Int      4       4

3       l_name                          char            8       8

4       l_dir                           char            24      16

5       l_dst                           8-byte float    8       40

6       junk                            char            104     48

152

Table Name:             ann_h_ele_circle

Table No.               3

Table condition:        2

Col.                                                            Byte

No.     Col. Name       Description     Col. Type       Length  Offset

1       type                            4-byte Int      4       0

2       unused                          4-byte Int      4       4

3       c_name                          char            8       8

4       c_delta                         char            24      16

5       c_doc                           char            24      40

6       c_dir                           char            24      64

7       radius                          8-byte float    8       88

8       arc                             8-byte float    8       96

9       tc                              8-byte float    8       104

10      chd                             8-byte float    8       112

11      mo                              8-byte float    8       120

12      ext                             8-byte float    8       128

13      junk                            char            16      136

152

Table Name:             ann_h_ele_spiral

Table No.               4

Table condition:        4

Col.                                                            Byte

No.     Col. Name       Description     Col. Type       Length  Offset

1       type                            4-byte Int      4       0

2       unused                          4-byte Int      4       4

3       s_name                          char    8       8

4       s_theta                         char    24      16

5       s_dir                           char    24      40

6       ent_r                           8-byte float    8       64

7       ext_r                           8-byte float    8       72

8       ls                              8-byte float    8       80

9       ks                              8-byte float    8       88

10      p                               8-byte float    8       96

11      ys                              8-byte float    8       104

12      xs                              8-byte float    8       112

13      lc                              8-byte float    8       120

14      st                              8-byte float    8       128

15      lt                              8-byte float    8       136

16      a                               8-byte float    8       144

152

Binary File Structure - rwy

Structure:	roadway_modeler

Table Name:	roadway_modeler

Table No.	1

Table condition:

Col.					Byte

No.	Col. Name	Description	Col. Type	Length	Offset

1	halg		char	40	0

2	valg		char	40	40

3	direction		8-byte float	8	80

4	t_c_name		char	16	88

5	station		char	32	104

6	dstation		8-byte float	8	136

7	x_coord		8-byte float	8	144

8	y_coord		8-byte float	8	152

9	z_coord		8-byte float	8	160

10	offset		8-byte float	8	168

11	ori_surf_elev		8-byte float	8	176

12	cut_fill_hgt		8-byte float	8	184

13	msg		char	40	192

14	pnt_was_supered		char	1	232

15	super_rate		8-byte float	8	233

16	template_name		char	32	241

17	temp_layer_name		char	32	273

305

Binary File Structure - vol

Structure:	end_area_vol

Table Name:	end_area_vol

Table No.	2

Table condition:

Col.					Byte

No.	Col. Name	Description	Col. Type	Length	Offset

1	orig_surface		char	32	0

2	orig_material		char	32	32

3	des_surf		char	32	64

4	des_mat		char	32	96

5	alignment		char	32	128

6	station		char	24	160

7	cut_area		8-byte float	8	184

8	fill_area		8-byte float	8	192

9	cut_vol		8-byte float	8	200

10	fill_vol		8-byte float	8	208

11	net_vol		8-byte float	8	216

12	cut_factor		8-byte float	8	224

13	fill_factor		8-byte float	8	232

14	adj_cut		8-byte float	8	240

15	adj_fill		8-byte float	8	248

16	adj_net		8-byte float	8	256

17	added_cut		8-byte float	8	264

18	added_fill		8-byte float	8	272

19	added_net		8-byte float	8	280

20	add_cutfactor		8-byte float	8	288

21	add_filfactor		8-byte float	8	296

22	adj_ad_cut_vol		8-byte float	8	304

23	adj_ad_fil_vol		8-byte float	8	312

24	adj_ad_net_vol		8-byte float	8	320

25	balance_factor		8-byte float	8	328

26	mass_ordinate		8-byte float	8	336

27	setid		4-byte int	4	344

28	unused		4-byte int	4	348

29	tsoil_cut		8-byte float	8	352

30	tsoil_fill		8-byte float	8	360

31	undercut		8-byte float	8	368

32	sta_intern		8-byte float	8	376

33	buf		char	8	384

392

Binary File Structure - xsec

Structure:	cross-section

Table Name:	cross_section

Table No.	1

Table condition:	0

Col.					Byte

No.	Col. Name	Description	Col. Type	Length	Offset

1	halg		char	64	0

2	direction		8-byte float	8	64

3	station		char	32	72

4	dstation		8-byte float	8	104

5	surf_name		char	32	112

6	offset		8-byte float	8	144

7	elev		8-byte float	8	152

8	x		8-byte float	8	160

9	y		8-byte float	8	168

10	z		8-byte float	8	176

11	message		char	40	184

12	material_name		char	32	224

256